| MT XXX |  |  |
| --- | --- | --- |
| Meta-Cognitive Reasoning for Alarm Flood Management in Industrial Systems |  |  |
| Ankita Sen st184745@stud.uni-stuttgart.de |  |  |
| Electrical Engineering |  |  |
| User Requirements Specification |  |  |
|  | Examiner: | Prof. Dr.-Ing. Dr. h. c. Michael Weyrich |
|  | Supervisor: | Peter Frank |

Document Version Management

| Version | Author | Date | Status | Changes |
| --- | --- | --- | --- | --- |
| 0.1 | Fr | 27.11 | in progress | Creation |
| 0.2 | Fr | 28.11 | In progress | Added Requirements |
| 1.0 | Fr | 01.12 | final | Added Requirements |

# Table of Contents

0	Table of Contents	2

1	Introduction into the Project	3

2	Objectives	3

3	Operational Area	5

4	Requirements to the Conception	5

5	Quality Requirements	6

6	Execution	6

7	Additions	7

8	Literature	7

# Introduction into the Project

Alarm flood management focuses on handling situations in which a large number of alarms are triggered within a short period, overwhelming human operators. In modern industrial plants, alarm systems are essential for monitoring process conditions in real time and for providing early warnings of hazardous or abnormal situations. When alarm floods occur, operators are often overloaded by the high volume of information, making it difficult to identify root causes and initiate appropriate corrective actions. This can lead to poor decision-making, production downtime, or even catastrophic events. Therefore, effective alarm flood management is crucial to ensure safety, process stability, and timely human response.[1]

Offshore Power-to-X (PtX) production, as investigated in the hydrogen lead project H2Mare[2], [3], represents a prime example of a highly complex industrial system. In this project, green hydrogen and other PtX products are generated directly from wind energy on offshore platforms without connection to the power grid. While the high availability and consistency of offshore wind offer the potential for cost-efficient energy conversion, the operational environment poses significant challenges. These systems must operate under harsh and highly dynamic environmental conditions, which increase the risk of component failures and abnormal situations. Furthermore, the remote location and limited access for maintenance require a high degree of automation, robustness, and reliable alarm handling to ensure safe and continuous operation. This makes offshore PtX platforms a relevant use case where effective alarm-flood management is essential to maintain process stability, safety, and timely response to critical events.

In previous works (for example in [4]), a digital twin for the teleoperation of offshore PtX plants was developed, including a prototype implementation of an alarm-flood management system. This system applies root-cause analysis based on alarm history to support the operator in resolving alarm floods by providing contextual information. The concept was evaluated on the popular Tennessee Eastman Benchmark Process first described in [5]. However, the evaluation revealed that conventional methods are only partially effective, particularly when dealing with complex or previously unseen alarm scenarios.

A promising way to overcome these limitations is to integrate meta-cognitive reasoning into alarm-flood management. Meta-cognition allows a system to reflect on and adapt its own reasoning, particularly under uncertainty or unfamiliar alarm situations. By recognizing when confidence in a diagnosis is low or when additional strategies or human input are required, a meta-cognitive system can respond more flexibly and reliably than fixed rule-based methods. This increases robustness to novel alarm patterns and supports more informed operator decisions, making meta-cognition a key enabler for safer and more efficient offshore PtX operations.

Figure 1: H2Mare Offshore PtX-Plattform

# Objectives

The aim of this master thesis is to enhance existing alarm flood management systems by integrating meta-cognitive reasoning capabilities. The first step is to study and analyze the state of the art in alarm flood management systems, agentic Large Language Model (LLM) systems, and metacognition. To this end, a systematic literature review will be conducted, focusing on current methods, benchmarks, and limitations in alarm flood management. Based on the findings, the potential benefits of extending these systems with meta-cognitive reasoning will be identified.

Building on the literature review, a concept for extending conventional alarm flood management systems with meta-cognitive reasoning will be developed. This concept will enable the system to evaluate and reflect on the outputs of standard alarm management processes, including assessing the quality and reliability of alarm detection. Additionally, the system will be designed to perform root cause analysis using process knowledge and real-time monitoring data, providing operators with valuable contextual information to support decision-making.

Optionally, the system can be enhanced with a decision-making module that determines how, and to what extent, it can autonomously manage alarm floods. If sufficient information is not available, it should determine strategies to obtain the required data. This may include dispatching an appropriate robot to gather missing information through manual inspection or interactively collaborating with the operator to resolve the alarm situation. If none of these strategies provide an adequate solution, the system should transfer control to the operator while supplying useful information. To achieve this, the system must also be capable of evaluating its proposed solutions based on appropriate parameters such as safety and feasibility.

Finally, the proposed concept will be implemented and evaluated using the Tennessee Eastman Process, with performance compared against suitable baseline methods. To assess generalizability, the system may also be evaluated on an additional benchmark process identified in the literature review.

Alarm flood management is grounded in industrial alarm management standards and operator‑centric workflows. Standards such as ISA‑18.2 define a lifecycle that spans alarm philosophy, rationalization, design, implementation, operation, monitoring, and audit, with the goal of reducing nuisance alarms and keeping alarm loads within manageable levels. Empirical studies in the process industry confirm that alarm overload and poor prioritization remain a leading contributor to delayed operator response and safety risk, highlighting the need for disciplined alarm design and continuous improvement. [1], [7], [11]

Beyond governance, current methods focus on data‑driven analysis of alarm floods and root causes. Pattern‑mining and association methods identify frequent alarm subsequences, while similarity measures and time‑series classification support the identification of recurring flood patterns and novelty. Recent work also explores causal analysis and deep‑learning‑based root‑cause reasoning for alarm sequences, but these approaches are often sensitive to sequence length, missing context, or shifts in operating conditions. Validation studies emphasize that similarity metrics must be carefully chosen and calibrated. [8], [9], [10], [13], [14], [15]

Digital‑twin‑based assistance systems provide a unifying context layer for alarm management by integrating process data, alarm history, and semantic metadata into a coherent model of the plant. In offshore Power‑to‑X applications, such a layer is particularly valuable due to remote operation, high maintenance costs, and limited operator exposure to rare fault scenarios. The digital‑twin architecture proposed for PtX teleoperation shows how alarm assistance can be embedded as a modular component, enabling real‑time monitoring and knowledge reuse. These capabilities motivate the integration of higher‑level reasoning and meta‑cognition on top of conventional alarm‑analysis pipelines. [3], [4]

In summary, the state of the art comprises three complementary pillars: (1) standards‑based lifecycle management to control alarm load, (2) data‑driven alarm sequence analysis to identify and classify alarm floods, and (3) digital‑twin infrastructures that provide contextual integration for decision support. This thesis builds on these pillars and extends them with meta‑cognitive reasoning to improve reliability in uncertain or novel alarm scenarios. [1], [4], [7], [13]

# Theory

## UR10 – Basis Technologies and State of the Art

# Operational Area

The results of this work are intended to be used in the future to optimize alarm flood management systems enabling a safer and autonomous operation of operation. It also serves a basis for continued research on meta-cognitive autonomy in the field of process automation.

# Requirements to the Conception

| /UR10/ | Familiarize yourself with the basis technologies and state of the art. |  |
| --- | --- | --- |
| /UR11/ | Representation and description of the fundamentals of Alarm flood management systems |  |
| /UR12/ | Representation and description of the fundamentals of Agentic LLMs |  |
| /UR13/ | Representation and description of the fundamentals of meta-cognition |  |
| /UR13/ | Conduct a literature review on alarm flood management systems focusing on current methods, benchmarks, and limitations in alarm flood management |  |
| /UR20/ | Develop a conceptual architecture for integrating meta-cognitive reasoning into conventional alarm flood management systems. |  |
| /UR21/ | Define mechanisms for system self-evaluation, including assessment of alarm detection quality and reliability. |  |
| /UR22/ | Design a meta-cognitive module for root cause analysis using available process knowledge and real-time monitoring data. |  |
| /UR23/ | Specify how contextual insights should be presented to support operator decision-making |  |
| /UR24/ | Specify interfaces between the modules and the system boundaries (e.g. to the standard alarm flood management components, the operator, additional resources) |  |
| /UR30/ | [Optional] Develop a concept for a decision-making module enabling autonomous alarm flood management. The decision module should be able to generate new strategies for solving unseen scenarios. |  |
| /UR40/ | Implementation of the concept. The implementation should be as modular as possible to allow future testing of additional approaches and to ensure easy reusability of individual code components. |  |
| /UR50/ | Evaluation of the implemented concept based on the Tennessee Eastman Process. For this purpose, a “standard” baseline model should be used to compare the system using appropriate metrics. |  |
| /UR51/ | [Optional] Evaluate the generalizability on a different dataset. |  |

UR10 – Basis technologies and state of the art: This work builds on established alarm management standards (e.g., ISA‑18.2 and IEC 62682), alarm flood analysis methods (pattern mining, similarity analysis, and root‑cause identification), and digital‑twin based assistance systems for process operation. It additionally considers the current state of agentic LLMs for contextual reasoning and meta‑cognitive self‑assessment. The review focuses on practical alarm‑management workflows, the Tennessee Eastman benchmark as a de‑facto testbed, and recent approaches to alarm sequence mining, alarm flood classification, and alarm rationalization.

# Quality Requirements

| Product Quality | very high | high | normal | not relevant |
| --- | --- | --- | --- | --- |
| Theory | x |  |  |  |
| Functionality |  |  | x |  |
| Usability |  |  | x |  |
| Applicability |  |  | x |  |
| Portability |  | x |  |  |

# Execution

The thesis has to be executed according to the “IAS Process Model” (Model for Conceptional Projects). In the following table, the documents which have to be created are listed.

Table 1: Documents to be created

| Project Phase |  |  |  | FA, MT |
| --- | --- | --- | --- | --- |
| CD 1 Definition Phase |  |  |  |  |
|  | PM 1 | Project Planning | Project Plan | X |
|  | CD 1.1 | Analysis of Requirements | System Requirements Specification | X |
|  | CD 1.2 | Analysis of the Basis | Basis | X |
| CD 2 Conception Phase |  |  |  |  |
|  | QA 2.1 | Creation of Evaluation Specification | Evaluation Specification | O |
|  | CD 2.1 | Creation of the Conception | Conception | X |
| CD 3 Prototyping Phase |  |  |  |  |
|  | CD 3.1 | Implementation of the Prototype | Prototype Description | X |
|  | QA 2.2 | Evaluation | Evaluation Protocol | X |
|  | PM 3 | Project Finish | Final Project Report | O |

| Legend: |  |  |
| --- | --- | --- |
| X |  | Document is mandatory |
| O |  | Document is optional |

The current state of the thesis and results have to be discussed with the tutor every two weeks.

The IAS guidelines have to be respected.

# Additions

None.

# Literature

[1]	F. E. Mustafa et al., “A review on effective alarm management systems for industrial process control: Barriers and opportunities,” Int. J. Crit. Infrastruct. Prot., vol. 41, p. 100599, Jul. 2023, doi: 10.1016/j.ijcip.2023.100599.

[2]	“Wasserstoff-Leitprojekte: H2Mare: Offshore-Technologien.” Accessed: Mar. 21, 2025. [Online]. Available: https://www.wasserstoff-leitprojekte.de/leitprojekte/h2mare

[3]	P. Häbig et al., “A Modular System Architecture for an Offshore Off-grid Platform for Climate-neutral Power-to-X Production in H2Mare,” Procedia CIRP, vol. 126, pp. 909–914, Jan. 2024, doi: 10.1016/j.procir.2024.08.348.

[4]	F. Bodenstein, S. Dieckmann, D. Dittler, A. Geschke, N. Jazdi, and M. Weyrich, “Digital Twin to Enhance Offshore Power-to-X Platforms with Operational Alarm Management,” Procedia CIRP, vol. 130, pp. 780–785, Jan. 2024, doi: 10.1016/j.procir.2024.10.164.

[5]	J. J. Downs and E. F. Vogel, “A plant-wide industrial process control problem,” Comput. Chem. Eng., vol. 17, no. 3, pp. 245–255, Mar. 1993, doi: 10.1016/0098-1354(93)80018-I.

[6]	RC-US DI PA MI PRM MS, “White Paper: Setting a New Standard in Alarm Management.”

[7]	isa.org, “ANSI/ISA-18.2-2016, Management of Alarm Systems for the Process Industries,” 2024. https://www.isa.org/products/ansi-isa-18-2-2016-management-of-alarm-systems-for (accessed 9 April 2024).

[8]	G. Ghasemi, D. Braun, et al., “A data-driven approach to analyze industrial process alarms using the association analysis method,” 2023, pp. 777–790.

[9]	N. Javanbakht, A. Neshastegaran, I. Izadi, “Alarm-Based Root Cause Analysis in Industrial Processes Using Deep Learning,” 2022.

[10]	V. Rodrigo, M. Chioua, et al., “Causal analysis for alarm flood reduction,” IFAC-PapersOnLine, vol. 49, 2016, pp. 723–728. https://doi.org/10.1016/j.ifacol.2016.07.269.

[11]	M. Bockelmann, P. Nickel, et al., “Empirische Studie zur Gestaltung von Alarmsystemen und Alarmmanagement in der Prozessindustrie,” Z. Arb. Wiss., vol. 73, 2019, pp. 91–99. https://doi.org/10.1007/s41449-018-00137-8.

[12]	NAMUR, “NAMUR Open Architecture,” 2024. https://www.namur.net/de/fokusthemen/namur-open-architecture.html (accessed 9 April 2024).

[13]	G. Manca, A. Fay, “Detection of Historical Alarm Subsequences Using Alarm Events and a Coactivation Constraint,” IEEE Access, vol. 9, 2021, pp. 46851–46873. https://doi.org/10.1109/ACCESS.2021.3067837.

[14]	G. Manca, A. Fay, “Identification of Industrial Alarm Floods Using Time Series Classification and Novelty Detection,” in: 2022 IEEE 20th International Conference on Industrial Informatics (INDIN), IEEE, 2022.

[15]	M. Fullen, P. Schüller, et al., “Validation of similarity measures for industrial alarm flood analysis,” pp. 93–109.